/*

* SPDX-FileCopyrightText: 2010-2022 Espressif Systems (Shanghai) CO LTD

*

* SPDX-License-Identifier: CC0-1.0

*/

//librerias

#include <stdio.h>

#include <math.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "driver/gpio.h"

#include "esp_timer.h"

#include "esp_log.h"

#include "esp_adc/adc_oneshot.h"

//constantes

#define PINSENSORTEMP 36

#define MASKSENSORTEMP 1ULL << PINSENSORTEMP

#define CHANNELPOT1 0

static const char* TAG = "MyModule";

//declaración funciones

void tareaSensorTemperatura( void *);

//handles

adc_oneshot_unit_handle_t h_cad1;

TaskHandle_t h_tareaSensorTemperatura;

void app_main(void)

{

//creación tareas

xTaskCreate( tareaSensorTemperatura, "tareaSensorTemperatura", 2048, NULL, 1, &h_tareaSensorTemperatura);

while(1)

{

vTaskDelay(pdMS_TO_TICKS(10000));

ESP_LOGI(TAG, "SOY EL MAIN");

}

}

//definición funciones tareas

void tareaSensorTemperatura( void * parametro)

{

//configurar pin gpio36 como entrada

gpio_config_t configGpio36 = {

.intr_type = GPIO_INTR_DISABLE,

.mode = GPIO_MODE_INPUT,

.pull_up_en =  GPIO_PULLUP_DISABLE,

.pull_down_en = GPIO_PULLDOWN_DISABLE,

.pin_bit_mask = MASKSENSORTEMP

};

gpio_config(&configGpio36);

adc_oneshot_unit_init_cfg_t configCAD1 = {

.clk_src = ADC_RTC_CLK_SRC_DEFAULT,

.ulp_mode = ADC_ULP_MODE_DISABLE,

.unit_id = ADC_UNIT_1

};

adc_oneshot_new_unit(&configCAD1, &h_cad1);

adc_oneshot_chan_cfg_t configChanPot1 = {

.atten = ADC_ATTEN_DB_0,

.bitwidth = ADC_BITWIDTH_9

};

adc_oneshot_config_channel(h_cad1, CHANNELPOT1, &configChanPot1);

int valorCAD;

while(1)

{

adc_oneshot_read(h_cad1, CHANNELPOT1, &valorCAD);

ESP_LOGI(TAG, "valor CAD: %d", valorCAD);

vTaskDelay(pdMS_TO_TICKS(3000));

}

}